Conservation Practice Effects

| Access Control (Ac) 472 Definition: The temporary or permanent exclusion of animals, people, vehicles, and/or equipment from an area. Lifespan: 10 Years. Major Resource Concerns Addressed: Soil erosion, streambank protection, wildlife habitat. Benchmark Condition: Riparian area. Date: 2025 Planner/Location: |  |
| --- | --- |
| Positive Effects | Negative Effects |
| Soil - Sheet, rill, wind, gully, streambank erosion is reduced by less disturbance of the soil and vegetation. - Organic matter increases. - Soil compaction is reduced. Water - Runoff, flooding, ponding, seasonal high water table and seeps may be reduced as plant vigor increase water uptake and soil structure improves. - Soil moisture will increase with improved vegetation and soil structure. - Nutrients and pesticides in surface and ground water will decrease as plant vigor improves soil conditions. - Pathogens and chemicals from manure in surface and groundwater will be reduced in sensitive areas. - Sediment in surface water will be reduced. - High water temperature will be reduced. - Petroleum, heavy metals and other pollutants in surface and groundwater will be reduced. Air - Emissions of particulate matter, ozone precursors and greenhouse gases will be reduced. Plants - Plant community productivity and health will increase by excluding animals, people, and vehicles and encouraging natural revegetation. - The threat of noxious and invasive plants may decrease. Animals - Fish and wildlife habitat, food, water, cover, and shelter will improve for certain wildlife species. - Fish and wildlife habitat continuity (space) will improve. - Livestock feed, forage and shelter will improve (if utilized in the future). Energy - Less energy will be expended with reduced land use. Human - Reduced labor and management. - Cultural resources will be protected. | Land - Land will be taken out of production with a change to less intense land use. Capital - Reduced use of field equipment. - Forgone income from previous land use. - Annual operation and maintenance costs to maintain control barrier. Labor - Labor requirements will decrease with land taken out of production. Management - Management requirements will be reduced with less effort managing livestock, people or other farm enterprises. Risk - Agricultural operation flexibility and timing will be limited due to deferred land use, reduced grazing or cropping. - Cash flow will decline, annual costs may be greater than annual benefits. - Profitability will decrease if land is taken out of production. - Wildfire hazard from excessive biomass accumulation may increase. - The threat of noxious and invasive plants may increase. |
| Net Effect: Access Control improves soil productivity, reduces erosion and protects wildlife in the long term at a moderate cost. |  |

Commonly Associated Practices: Prescribed Grazing (Code 528), Tree/Shrub Establishment (Code 612), Vegetative Treatment Area (Code 635), and Wetland Creation (Code 658).

Note: This worksheet contains general talking points for the conservation planner to discuss with the land user.  It is the first step towards an economic or financial analysis.  The second step would include identifying a specific site for analysis at the farm or field level, editing the template for local conditions, adding units and quantities of farm inputs and outputs.  The third step in the economic analysis is to place a dollar value on as many variables as possible, put all units in the same time frame, using amortization ($/Acres/Year) or net present value ($/Acre), so benefits and costs can be compared.  The fourth and final step would be to combine several conservation practices into a conservation system, which is how most conservation practices are applied at the field level. Data for the worksheet comes from the land user, conservation planner, technical specialist and local agricultural supply vendors and contractors.  See Economics Technical Note: TN 200-ECN-1, Basic Economic Analysis Using T-Charts for more information.