Conservation Practice Effects

| Contour Farming (Ac) 330 Definition: Aligning ridges, furrows, and roughness formed by tillage, planting and other operations at a grade near the contour to alter the velocity or the direction of water flow. Lifespan: 5 Years. Major Resource Concerns Addressed: Soil erosion and water quality. Benchmark Condition: Highly erodible steep cropland. Date: 2025 Developer/Location: |  |
| --- | --- |
| Positive Effects | Negative Effects |
| Soil - Reduced sheet, rill and gully erosion as contouring reduces runoff velocities and changes overland flow direction and reducing the detachment and transport capacity of over-land flow. - Reduced soil erosion increases organic matter. Water - Increased water infiltration will slightly reduce the potential for flooding, ponding and seasonal high water table. - Improved water storage in the profile. - Reduced runoff of pesticides, nutrients and pathogens into surface water. - Reduced transport of sediment to surface water. Air - No Change. Plants - Reduced erosion and sediment damage to crop yields. - Improved plant productivity and health with increased soil moisture. Animals - No Change. Energy - No energy savings. Human - Increase yields/reduce costs as land becomes more productive. - Create sustainability of natural resources that support your business. - Increase the property value (real estate) of your property. - Conserve soil and water for periods of drought and future use. - Prevent off-site negative impacts. - Comply with environmental regulations. - Save time, money and labor. - Promote family health and safety. - Make land more attractive and promote good stewardship. - May be eligible for cost share. - Increased profitability in the long run. | Land - No change in land use. - Possible change in land in production may if corners and end pieces taken out of production. Capital - No additional field equipment required. - Operation and maintenance costs. Labor - Increase in labor to annually maintain contour. Management - Increase time managing crop production. Risk - Decrease in agricultural operation flexibility and timing following designed row pattern. - Increase seeps with improved water infiltration that may move laterally to a seep area, particularly during fallow periods. - Possible increase of pesticides, nutrients, pathogens and agricultural chemicals into groundwater with increased infiltration. - Increase in equipment and energy use. |
| Net Effect: Improved soil productivity and water quality at a low cost. |  |

Commonly Associated Practices: Residue and Tillage Management, Reduced Till (Code 345) and No-Till (Code 329); Contour Buffer Strips (Code 332), Grassed Waterway (Code 412), Water and Sediment Control Basin (Code 638), and Underground Outlet (Code 620).

Note: This worksheet contains general talking points for the conservation planner to discuss with the land user.  It is the first step towards an economic or financial analysis.  The second step would include identifying a specific site for analysis at the farm or field level, editing the template for local conditions, adding units and quantities of farm inputs and outputs.  The third step in the economic analysis is to place a dollar value on as many variables as possible, put all units in the same time frame, using amortization ($/Acres/Year) or net present value ($/Acre), so benefits and costs can be compared.  The fourth and final step would be to combine several conservation practices into a conservation system, which is how most conservation practices are applied at the field level. Data for the worksheet comes from the land user, conservation planner, technical specialist and local agricultural supply vendors and contractors.  See Economics Technical Note: TN 200-ECN-1, Basic Economic Analysis Using T-Charts for more information.