Conservation Practice Effects

| Stormwater Runoff Control (Ac) 570 Definition: Measures or systems to control the quantity and quality of stormwater runoff. Lifespan: 1 Year. Major Resource Concerns Addressed: Water quantity, flooding. Benchmark Condition: Overland water flow floods headquarters. Date: 2025 Planner / Location: |  |
| --- | --- |
| Positive Effects | Negative Effects |
| Soil - Sheet, rill and gully erosion reduced by intercepting runoff water and shorten the slope length. - Collects and slows run-off to a non-erosive velocity, reducing ditch-bank erosion. Water - Diversion may provide outlet for seepage, intercept runoff and act as a floodway. - Reduced runoff, flooding, and ponding as water is diverted. - Intercepts shallow subsurface flows and reduces seasonal high water table. - May help capture and reuse runoff for irrigation, water-spreading or water-harvesting systems. - Water may be diverted from the pesticide or manure application site. - Some sediment is trapped, reducing the amount of sediment-adsorbed nutrients delivered off-site. Air - None. Plants - Reduced flooding may increase crop yields. Animals - None. Energy - None. Human - Increased farm efficiency by eliminating wet areas near headquarters. - Improved agricultural operation flexibility and timing with improved water control. - Diversion may protect historic properties from erosion. - Less irrigation labor required. - Improved opportunities for land use and water management. - Increase yields/reduced costs as land becomes more productive. - Create sustainability of natural resources that support your business. - Increase the property value (real estate) of your property. - Conserve soil and water for periods of drought and future use. - Prevent off-site negative impacts. - Comply with environmental regulations. - Save time, money and labor. - Promote family health and safety. - Make land more attractive and promote good stewardship. - May be eligible for cost share. - Increased profitability in the long run. | Land - Cultural resources may be damaged during construction. - No change in land use. - Minor amount of land taken out of agricultural production. Capital - No additional field equipment required. - Installation equipment. - Materials and on-site equipment. - Annual operation and maintenance costs to clean-out debris, repair and replace equipment and structure. Labor - Additional labor required to operate and maintain water control structures. Management - Monitoring stormwater runoff. Risk - May provide a source of seepage water. - Because diversions concentrate overland flows, there can be an increase in nutrients, pesticides, pathogens, manure and or other agricultural chemicals offsite. - Increased infiltration may provide transport for nutrients, pesticides, pathogens, manure and or other agricultural chemicals to ground water. - Increased compaction from construction equipment. - Increase in seepage because of controlled runoff may increase infiltration. |
| Net Effect: Improved water delivery and management at a moderate cost. |  |

Commonly Associated Practices: Dam (Code 402), Pond (Code 378), Diversion (Code 362), Grade Stabilization Structure (Code 410), Grassed Waterway (Code 412), Structure for Water Control (Code 587), Water and Sediment Control Basin (Code 638), Access Control (Code 472), and Critical Area Planting (Code 342).

Note: This worksheet contains general talking points for the conservation planner to discuss with the land user.  It is the first step towards an economic or financial analysis.  The second step would include identifying a specific site for analysis at the farm or field level, editing the template for local conditions, adding units and quantities of farm inputs and outputs.  The third step in the economic analysis is to place a dollar value on as many variables as possible, put all units in the same time frame, using amortization ($/Acres/Year) or net present value ($/Acre), so benefits and costs can be compared.  The fourth and final step would be to combine several conservation practices into a conservation system, which is how most conservation practices are applied at the field level. Data for the worksheet comes from the land user, conservation planner, technical specialist and local agricultural supply vendors and contractors.  See Economics Technical Note: TN 200-ECN-1, Basic Economic Analysis Using T-Charts for more information.